ENVIRONMENTAL STATEMENT

FOR THE YEAR 2024-2025

AIZAWL MUNICIPAL CORPORATION

SUBMITTED BY

AIZAWL MUNICIPAL CORPORATION

ENVIRONMENTAL CELL

AIZAWL, MIZORAM.

ANNEXURE

ENVIRONMENTAL STATEMENT FORM-V

(See rule 14)

Environmental Statement for the financial year ending with 31 st March

PART-A

(i) Name and address of the owner/ occupier of the industry operation or process

|  |  | AIZAWL MUNICIPAL CORPORATION |
| --- | --- | --- |
| (ii) | Industry category | Municipal Waste Management Center |
|  | Primary-(STC Code) | NA |
|  | Secondary- (STC Code) | √ |
| (iii) | Production category - Units | NA |
| (iv) | Year of establishment | 11th Dec 2019 |
| (v) | Date of the last environmental statement submitted | 5th June 2024 |

PART -B

Water and Raw Material Consumption:

- Water consumption in m3/d

Process:	NA

Cooling:	NA

Domestic:	Only for drinking (1.0 KLD)

- Raw material consumption

| Name of raw materials* | Name of Products | Consumption of raw material per unit of output |  |
| --- | --- | --- | --- |
|  |  | During the previous financial year | During the current financial year |
| Municipal solid waste | Waste material for recycling | 5.92 tons per day | 11.57 TPD |

* Industry may use codes if disclosing details of raw material would violate contractual obligations, otherwise all industries have to name the raw materials used.

PART-C

Pollution discharged to environment/unit of output

(Parameter as specified in the consent issued)

| Pollutants | Quantity of Pollutants discharged (mass/day) | Concentration of Pollutants discharged (mass/volume) | Percentage of variation from prescribed standards with reasons |
| --- | --- | --- | --- |
| (a)Water | 500 litres/day | S. No; Parameter; Standards ( Mode of Disposal ); Results. Land disposal. 1.; Suspended solids, mg/l, max; 200; 130. 2.; Dissolved solids (inorganic) mg/l, max.; 2100; 790. 3; pH value; 5.5 to 9.0; 7.3. 4; Ammonical nitrogen (as N), mg/l, max.; -; 38. 5; Total Kjeldahl nitrogen (as N), mg/l, max.; -; 75. 6; Biochemical oxygen demand (3 days at 270 C) max.(mg/l); 100; 51. 7; Chemical oxygen demand, mg/l, max.; -; 140. 8; Arsenic (as As), mg/l, max; 0.2; 0.15. 9; Mercury (as Hg), mg/l, max; -; 0.009. 10; Lead (as Pb), mg/l, max; -; 0.09. 11; Cadmium (as Cd), mg/l, max; -; 0.005. 12; Total Chromium (as Cr), mg/l, max.; -; 0.15. 13; Copper (as Cu), mg/l, max.; -; 2.5. 14; Zinc (as Zn), mg/l, max.; -; 4. 15; Nickel (as Ni), mg/l, max; -; 2.5. 16; Cyanide (as CN), mg/l, max.; 0.2; BDL. 17; Chloride (as Cl), mg/l, max.; 600; 300. 18; Fluoride (as F), mg/l, max; -; 1.5. 19; Phenolic compounds (as C6H5OH) mg/l, max.; -; 0.5. As of 10th March 2025 | Within Standard |
| (b)Air | Continuous 8 hours | PM 2.5: 7 ppm PM 10: 55 ppm SO2: 2 ppm NOX: 16.9 ppm CO: 5 ppm As of 10th March 2025 | Within Standard |

PART-D

HAZARDOUS WASTES

(as specified under Hazardous Wastes (Management & Handling Rules, 1989).

| Hazardous Wastes | Total Quantity (Kg) |  |
| --- | --- | --- |
|  | During the previous financial yea | During the current financial year |
| From Process | Negligible | Negligible |
| From Pollution Control Facilities | Negligible | Negligible |

PART- E

SOLID WASTES:

| Solid Wastes | Total Quantity (Kg) |  |
| --- | --- | --- |
|  | During the previous financial year | During the current financial year |
| a. From process | NA |  |
| b. From Pollution Control Facility | NA |  |
| c.Quantity recycled or reutilised within the unit. | Recycled waste in Quintals: Plastic waste bailed = 685.35 Mechanical Compost = N.A Metals = 87.60 Papers = 7540.02 Plastics = 13289.21 Total = 21602.18qtls 5.92 tons per day | Recycled waste in Quintals: Plastic waste bailed = 15,678.10 Mechanical Compost = 1,268.25 Metals = 176.22 Papers = 10,416.27 Plastics = 14,695.41 Total = 42,234.25 |

PART-F

Please specify the characteristics (in terms of concentration and quantum) of hazardous as well as solid wastes and indicate disposal practice adopted for both these categories of wastes.

Recyclable Wastes are segregated and collected for bailing. The bailed units are transported to other states for recycling process. There is no hazardous waste produced in the facility. Wet wastes are decomposed in a Mechanical-Composting Unit while other non-recyclable wastes are dumped in the Temporary Holdings due to reconstruction of the Land fill.

PART-G

Impact of the pollution control measures taken on conservation of natural resources and consequently on the cost of production.

| Sl.No | Particulars | Impact |
| --- | --- | --- |
| 1 | Leachate Management System | Improves the quality of leachate discharge in the environment. Subsequently improves the concentration of pollutants discharged. |
| 2 | Maintaining Greenbelt Area | Subsequently improves the quality of air in and around the facility. The air quality of the Centre is within prescribed standards. |

PART-H

Additional measures/investment proposal for environmental protection including abatement of pollution.

- Quarterly Monitoring of the Solid Waste Management Centre has been carried out regularly. Reports were submitted to SEIAA Mizoram and IRO Shillong.

- Construction of Additional Leachate Management System was carried out.

- Drainage system for Surface Water (for collection of surface water run-off) is under construction.

PART-I

MISCELLANEOUS: